Module 1

Carbon Accounting: GHG Emission Scopes, Emission Factors & Calculation Framework

A structured reference covering emission scope classification, official emission factors, calculation specifications, developer function specifications, and the validation & testing framework used in Carbonomics AI.

## Contents

- 1. Understanding Greenhouse Gas (GHG) Emission Scopes

- 2. Collecting Official Emission Factors

- 3. Carbon Calculation Specification

- 4. Developer Function Specification

- 5. Validation & Testing Framework

- 6. Module 1 Deliverables

# 1. Understanding Greenhouse Gas (GHG) Emission Scopes

## What Are GHG Emission Scopes?

Greenhouse Gas (GHG) emissions are classified into three scopes under the GHG Protocol, the most widely accepted international framework for greenhouse gas accounting. These scopes help organizations identify the source of emissions and avoid double counting while preparing carbon footprint inventories.

The classification ensures that emissions are categorized based on whether they are produced directly by the organization or indirectly through its activities.

## Scope 1 — Direct Emissions

### Definition

Scope 1 includes direct greenhouse gas emissions from sources that are owned or controlled by the organization. These emissions occur because the institution itself burns fuel or releases greenhouse gases during its operations.

### Common Sources

- Diesel generators

- Institute-owned buses

- Company-owned vehicles

- Petrol and diesel consumption

- LPG used in kitchens

- Refrigerant leakage from air conditioners

- Boilers and furnaces

### Example

Suppose a college operates two diesel generators during power cuts.

- Diesel consumed = 1,500 litres

- Diesel emission factor = 2.68 kg CO₂/L

Carbon emissions = 1,500 × 2.68 = 4,020 kg CO₂

This is a Scope 1 emission because the diesel is consumed directly by the institution.

### In Carbonomics AI

Scope 1 data may include:

- Diesel consumption

- Petrol consumption

- Generator fuel

- Institutional vehicle fuel records

## Scope 2 — Indirect Emissions from Purchased Electricity

### Definition

Scope 2 includes emissions generated during the production of electricity, steam, heating, or cooling that the organization purchases. Although the institution does not generate these emissions directly, it is responsible because it consumes the electricity.

### Common Sources

- Electricity purchased from the state electricity board

- Purchased steam

- Purchased chilled water

### Example

- Electricity consumed = 2,50,000 kWh

- CEA emission factor = 0.716 kg CO₂/kWh

Carbon emissions = 250,000 × 0.716 = 179,000 kg CO₂

Although the emissions occur at the thermal power plant, they are counted under the institution's Scope 2 emissions.

### In Carbonomics AI

Scope 2 will mainly include:

- Monthly electricity bills

- Annual electricity consumption

- Smart meter readings

This is generally one of the easiest datasets to collect because institutions already maintain electricity records.

## Scope 3 — Other Indirect Emissions

### Definition

Scope 3 includes all indirect emissions that occur because of the institution's activities but are neither owned nor directly controlled by the organization. These emissions usually represent the largest share of an institution's carbon footprint.

### Common Sources

- Student commuting

- Employee commuting

- Air travel

- Waste disposal

- Paper consumption

- Water treatment

- Purchased goods

- Food supply chain

### Example

- 1,000 students travel

- Average distance = 20 km/day

- Bus emission factor = 0.089 kg CO₂/passenger-km

The emissions are calculated based on passenger-kilometres. Even though the buses are not owned by the institution, these emissions are generated because students travel to attend college — hence they belong to Scope 3.

### In Carbonomics AI

Scope 3 includes:

- Student transport survey

- Employee commuting

- Waste generation

- Water consumption

- Sewage treatment

- Paper usage (if data available)

### Why Is Scope 3 Usually the Largest?

For educational institutions, Scope 3 often contributes more than 70% of total emissions because:

- Thousands of students travel daily.

- Employees commute from different locations.

- Large quantities of waste are generated.

- Water treatment and purchased materials add indirect emissions.

Your NED University case study also demonstrates this, where Scope 3 accounted for the majority of campus emissions, highlighting the importance of considering indirect emissions in institutional carbon accounting.

### Scope Comparison

| Scope | Type | Controlled by Institution? | Examples |
| --- | --- | --- | --- |
| Scope 1 | Direct emissions | Yes | Diesel generators, institute buses, LPG, fuel consumption |
| Scope 2 | Indirect emissions from purchased energy | Partially | Electricity purchased from the grid |
| Scope 3 | Other indirect emissions | No | Student transport, employee commuting, waste, water, paper, business travel |

# 2. Collecting Official Emission Factors

## Why Are Emission Factors Needed?

An emission factor is a scientifically determined value that converts an activity into its equivalent greenhouse gas emissions.

Carbon Emissions (kg CO₂e) = Activity Data × Emission Factor

### Example

- Electricity consumed = 1,000 kWh

- Emission factor = 0.716 kg CO₂e/kWh

Carbon Emissions = 1,000 × 0.716 = 716 kg CO₂e

## Priority of Sources (Very Important)

Always follow this order — it will make your report much stronger during evaluation.

| Priority | Source | Why? |
| --- | --- | --- |
| 1 | Central Electricity Authority (CEA), India | Official Indian electricity emission factor |
| 2 | IPCC 2006 / 2019 Guidelines | International emission factors |
| 3 | GHG Protocol | Scope definitions and accounting methodology |
| 4 | DEFRA (UK Government) | Transport & waste factors if Indian data unavailable |
| 5 | Peer-reviewed Research Papers | Only when official factors are unavailable |

## Emission Factor Master Table

### 1. Electricity

Activity: Electricity Consumption

Unit: kWh

Emission Factor: 0.716 kg CO₂e/kWh (use the latest CEA value applicable to your reporting year; update if a newer official factor is adopted)

Scope: Scope 2

Source: Central Electricity Authority (CEA), India

CO₂e = Electricity (kWh) × EF

### 2. Diesel

Unit: Litre

Emission Factor: 2.68 kg CO₂e/L

Scope: Scope 1

Source: IPCC / GHG Protocol

CO₂e = Diesel (L) × 2.68

### 3. Petrol

Unit: Litre

Emission Factor: 2.31 kg CO₂e/L

Scope: Scope 1

Source: IPCC / GHG Protocol

CO₂e = Petrol (L) × 2.31

### 4. LPG

Unit: kg

Emission Factor: ≈ 3.0 kg CO₂e/kg LPG

Scope: Scope 1

Source: IPCC

## Transport Factors

Transport emissions are generally calculated using passenger-km.

| Mode | Unit | Emission Factor | Scope |
| --- | --- | --- | --- |
| Bus | Passenger-km | 0.089 kg CO₂e/passenger-km | Scope 3 |
| Car | Passenger-km | 0.171 kg CO₂e/passenger-km | Scope 3 |
| Motorcycle | Passenger-km | 0.103 kg CO₂e/passenger-km | Scope 3 |
| Auto Rickshaw | Passenger-km | Use a documented factor from an Indian / recognized transport emissions source; avoid estimating without citation | Scope 3 |
| Bicycle / Walking | Passenger-km | 0 | — |

## Waste

For institutional waste, the exact factor depends on the disposal route: landfill, composting, recycling, or incineration.

If your dataset only contains total municipal solid waste, use a single documented factor and clearly state the disposal assumption in the report.

Unit: kg Waste

Scope: Scope 3

Source: IPCC / DEFRA / CPCB (depending on the factor adopted)

## Water

Water emissions are associated with treatment and pumping.

Unit: m³ (or kL)

Scope: Scope 3

Source: DEFRA / IPCC, or local utility methodology if available

## Master Excel Format

Consolidated reference table combining all emission sources, categories, scopes, factors, and formulas.

| # | Source / Activity | Category | Scope | Unit | Emission Factor | Formula | Reference | Remarks |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | Grid Electricity (India) | Energy | 2 | kWh | 0.7117 kg CO₂e/kWh | Electricity × EF | CEA | Purchased grid electricity |
| 2 | Diesel (Generator) | Fuel | 1 | Litre | 2.68 kg CO₂e/L | Diesel × EF | IPCC 2006 | Diesel generators / institution-owned equipment |
| 3 | Petrol Vehicle | Transport | 3 | km | 0.192 kg CO₂e/km | Distance × EF | DEFRA 2024 | Student/employee petrol vehicles |
| 4 | Diesel Vehicle | Transport | 3 | km | 0.171 kg CO₂e/km | Distance × EF | DEFRA 2024 / IPCC | Student/employee diesel vehicles |
| 5 | Electric Vehicle (EV) | Transport | 3 | kWh | 0.716 kg CO₂e/kWh | Electricity × EF | CEA India | EV charging emissions |
| 6 | College Bus* | Transport | 1 / 3 | km | 0.822 kg CO₂e/km | Distance × EF | GHG Protocol | Scope 1 if owned, Scope 3 if outsourced |
| 7 | Public Transport Bus | Transport | 3 | Passenger-km | 0.105 kg CO₂e/passenger-km | Passenger-km × EF | DEFRA 2024 | Public buses used by students/employees |
| 8 | Motorcycle | Transport | 3 | Passenger-km | 0.103 kg CO₂e/passenger-km | Passenger-km × EF | DEFRA | Two-wheelers |
| 9 | Auto Rickshaw | Transport | 3 | Passenger-km | TBD | Passenger-km × EF | CPCB / Research Paper | Final factor to be approved |
| 10 | Bicycle | Transport | 3 | Passenger-km | 0 kg CO₂e/passenger-km | Passenger-km × 0 | GHG Protocol | Zero operational emissions |
| 11 | Walking | Transport | 3 | Passenger-km | 0 kg CO₂e/passenger-km | Passenger-km × 0 | GHG Protocol | Zero operational emissions |
| 12 | Municipal Solid Waste (Landfill) | Waste | 3 | kg | 1.90 kg CO₂e/kg | Waste × EF | IPCC 2006 Vol. 5 | Landfill disposal assumption |
| 13 | Food Waste (Composting) | Waste | 3 | kg | 0.10 kg CO₂e/kg | Waste × EF | IPCC 2006 | If composting is adopted |
| 14 | Water Consumption | Water | 3 | m³ | TBD | Water × EF | DEFRA / IPCC | Final factor to be approved |
| 15 | Sewage Treatment (CH₄) | Wastewater | 3 | kg CH₄ | 28 kg CO₂e/kg CH₄ | CH₄ × 28 | IPCC AR6 | Global Warming Potential |
| 16 | Sewage Treatment (N₂O) | Wastewater | 3 | kg N₂O | 265 kg CO₂e/kg N₂O | N₂O × 265 | IPCC AR6 | Global Warming Potential |

### Factors Pending Finalization

| Source / Activity | Scope | Unit | Status |
| --- | --- | --- | --- |
| Motorcycle | 3 | Passenger-km | To be finalized |
| Auto Rickshaw | 3 | Passenger-km | To be finalized |
| Bicycle | 3 | Passenger-km | EF = 0 |
| Walking | 3 | Passenger-km | EF = 0 |
| Water Supply | 3 | m³ | To be finalized |
| Sewage Treatment | 3 | m³ | To be finalized |
| Refrigerant Leakage (if AC data is available) | 1 | kg refrigerant | Depends on refrigerant type |

# 3. Carbon Calculation Specification

This section documents each emission calculation: description, scope, required inputs, formula, and a worked example.

## 3.1 Electricity Emissions

Calculates indirect greenhouse gas emissions due to purchased grid electricity consumed by the institution.

Scope: Scope 2

Activity Data Required:

- Electricity Consumption (kWh)

Emission Factor: 0.7117 kg CO₂e/kWh

CO₂e = Electricity (kWh) × 0.7117

Input: electricity_kwh

Output: Electricity Emissions (kg CO₂e)

Example:

Electricity Consumed = 10,000 kWh

CO₂e = 10,000 × 0.7117 = 7,117 kg CO₂e

Validation Rules:

- Electricity cannot be negative.

- Unit must be kWh.

- Empty values should return an error.

## 3.2 Diesel Consumption

Calculates direct emissions produced by diesel generators or institution-owned diesel vehicles.

Scope: Scope 1

Activity Data Required:

- Diesel Consumed (Litres)

Emission Factor: 2.68 kg CO₂e/L

CO₂e = Diesel (L) × 2.68

Input: diesel_litres

Output: Diesel Emissions (kg CO₂e)

Example:

Diesel = 500 L

CO₂e = 500 × 2.68 = 1,340 kg CO₂e

Validation Rules:

- Litres ≥ 0

- Decimal values allowed

- Unit must be litres

## 3.3 Petrol Vehicle Emissions

Calculates emissions from petrol-powered vehicles.

Scope: Scope 3 (or Scope 1 if institution-owned vehicles are considered)

Activity Data Required:

- Distance Travelled (km)

Emission Factor: 0.192 kg CO₂e/km

CO₂e = Distance (km) × 0.192

Example:

Distance = 100 km

CO₂e = 100 × 0.192 = 19.2 kg CO₂e

## 3.4 Diesel Vehicle Emissions

Activity Data Required:

- Distance travelled (km)

CO₂e = Distance (km) × 0.171

Example:

200 km → 200 × 0.171 = 34.2 kg CO₂e

## 3.5 Electric Vehicle Emissions

CO₂e = Electricity Used (kWh) × 0.716

## 3.6 College Bus Emissions

CO₂e = Distance (km) × 0.822

Example:

50 km/day → 50 × 0.822 = 41.1 kg CO₂e

## 3.7 Public Transport Bus

Passenger-km = Passengers × Distance

CO₂e = Passenger-km × 0.105

Example:

100 students × 10 km = 1,000 passenger-km

1,000 × 0.105 = 105 kg CO₂e

## 3.8 Waste Emissions

CO₂e = Waste (kg) × 1.9

Example:

100 kg waste → 190 kg CO₂e

## 3.9 Wastewater (Methane)

CO₂e = CH₄ × 28

## 3.10 Wastewater (Nitrous Oxide)

CO₂e = N₂O × 265

# 4. Developer Function Specification

## 4.1 Common Rules (Applicable to All Functions)

- Input values cannot be negative.

- Accept integers and decimal values.

- Empty or null values should return an error message.

- All calculations should return emissions in kg CO₂e.

- Round the output to 2 decimal places.

## 4.2 Function: calculate_electricity_emissions()

Calculates Scope 2 emissions from purchased electricity.

Inputs:

- electricity_kwh — float, kWh

CO₂e = electricity_kwh × 0.7117

Returns: kg CO₂e

Input: 1000 → Output: 711.70

## 4.3 Function: calculate_diesel_emissions()

Inputs:

- diesel_litres — float, L

Diesel × 2.68

Returns: kg CO₂e

Input: 500 → Output: 1340.00

## 4.4 Function: calculate_petrol_vehicle_emissions()

Inputs:

- distance_km — float, km

distance × 0.192

Returns: kg CO₂e

## 4.5 Function: calculate_diesel_vehicle_emissions()

distance × 0.171

Returns: kg CO₂e

## 4.6 Function: calculate_ev_emissions()

electricity_used × 0.716

Returns: kg CO₂e

## 4.7 Function: calculate_college_bus_emissions()

distance × 0.822

Returns: kg CO₂e

## 4.8 Function: calculate_public_bus_emissions()

Inputs:

- passengers — int/float

- distance — float, km

passenger_km = passengers × distance

CO₂ = passenger_km × 0.105

Returns: kg CO₂e

## 4.9 Function: calculate_waste_emissions()

waste × 1.9

Returns: kg CO₂e

## 4.10 Function: calculate_methane_emissions()

methane × 28

Returns: kg CO₂e

## 4.11 Function: calculate_nitrous_oxide_emissions()

nitrous_oxide × 265

Returns: kg CO₂e

## 4.12 Master Function

Instead of calling every function separately, the backend should expose one master function that coordinates all calculations: calculate_total_emissions()

Inputs:

- Electricity

- Diesel

- Petrol Distance

- Diesel Distance

- EV Electricity

- College Bus Distance

- Public Bus Passenger-km

- Waste

- CH₄

- N₂O

Processing:

1. Call each individual calculation function.

2. Compute emissions for every category.

3. Sum all values.

4. Group emissions by Scope 1, Scope 2, and Scope 3.

Output Structure:

{

"scope1": value,

"scope2": value,

"scope3": value,

"total": value

}

## 4.13 Error Handling

| Condition | Expected Response |
| --- | --- |
| Negative value | Raise ValueError with an informative message |
| Empty input | Return validation error |
| Invalid data type | Raise TypeError |
| Zero input | Return 0.0 kg CO₂e |

## 4.14 Coding Standards

- Keep one function per emission source.

- Store emission factors in a separate configuration file (emission_factors.json or emission_factors.py) instead of hardcoding them throughout the code.

- Use descriptive function and variable names.

- Add docstrings to every function.

- Write unit tests for each calculation function.

# 5. Validation & Testing Framework

## 5.1 Validation Workflow

1. Input Data

2. Manual Excel Calculation

3. Python Calculation

4. Compare Results

5. Difference ≤ 0.01 kg CO₂e? → Yes: PASS  /  No: Investigate Error

## 5.2 Test Cases

| # | Test Case | Input | Expected Calculation | Expected Output |
| --- | --- | --- | --- | --- |
| 1 | Electricity | 1000 kWh | 1000 × 0.7117 | 711.70 kg CO₂e |
| 2 | Diesel | 500 L | 500 × 2.68 | 1340.00 kg CO₂e |
| 3 | Petrol Vehicle | 100 km | 100 × 0.192 | 19.20 kg CO₂e |
| 4 | Diesel Vehicle | 250 km | 250 × 0.171 | 42.75 kg CO₂e |
| 5 | Public Transport | 100 pax × 20 km = 2000 pkm | 2000 × 0.105 | 210.00 kg CO₂e |
| 6 | Waste | 150 kg | 150 × 1.9 | 285.00 kg CO₂e |
| 7 | Methane | 5 kg CH₄ | 5 × 28 | 140.00 kg CO₂e |
| 8 | Nitrous Oxide | 3 kg N₂O | 3 × 265 | 795.00 kg CO₂e |

## 5.3 Boundary Testing

| Test | Input | Expected Result |
| --- | --- | --- |
| Zero Electricity | 0 | 0 kg CO₂e |
| Zero Diesel | 0 | 0 kg CO₂e |
| Decimal Diesel | 12.75 L | Correct decimal result |
| Large Electricity | 1,000,000 kWh | No overflow |
| Large Waste | 50,000 kg | Correct calculation |

## 5.4 Invalid Input Testing

| Input | Expected Behaviour |
| --- | --- |
| -100 | Reject input |
| Empty value | Show validation error |
| Text ("abc") | Invalid data type |
| Null | Required field error |

## 5.5 Cross Validation

1. Calculate manually in Excel.

2. Calculate using the Python module.

3. Compare both outputs.

4. Record the difference.

Example validation sheet:

| Activity | Manual (kg CO₂e) | Software (kg CO₂e) | Difference | Status |
| --- | --- | --- | --- | --- |
| Electricity | 711.70 | 711.70 | 0.00 | PASS |
| Diesel | 1340.00 | 1340.00 | 0.00 | PASS |
| Petrol | 19.20 | 19.20 | 0.00 | PASS |

## 5.6 Acceptance Criteria

The module can be approved only if:

- All calculations match manual calculations within ±0.01 kg CO₂e.

- All validation test cases pass.

- Invalid inputs are correctly rejected.

- Scope classification is correct.

- Emission factors match the approved master table.

- Unit tests pass successfully.

## 5.7 Regression Testing

Whenever an emission factor or formula is updated:

1. Re-run all predefined test cases.

2. Verify no previously passing calculations fail.

3. Update the emission factor version in the documentation.

4. Obtain sign-off before deployment.

# 6. Module 1 Deliverables

At the end of Module 1, your repository should contain:

Module_1_Carbon_Accounting/

│

├── README.md

├── emission_factors.xlsx

├── emission_factors.json

├── calculation_specification.pdf

├── developer_function_specification.pdf

├── validation_test_cases.xlsx

├── manual_calculations.xlsx

├── references/

│   ├── CEA.pdf

│   ├── IPCC.pdf

│   ├── DEFRA.pdf

│   └── GHG_Protocol.pdf

├── src/

│   ├── emission_factors.py

│   ├── calculations.py

│   └── validator.py

└── tests/

├── test_electricity.py

├── test_transport.py

├── test_waste.py

└── test_total_emissions.py